AS170 Brick + Mortar Saw

SERVICE MANUAL

www.arbortech.com.au | www.arbortech-europa.de | www.arbortechusa.com

Rev.:2/10/2018

TABLE OF CONTENTS

1. INTRODUCTION

2. SAFETY

3. SPECIFICATIONS

4. DUST EXTRACTION ACCESSORY

5. SPECIAL TOOLS REQUIRED

6. INSPECTION AND REPAIR PROCEDURES

7. MANUFACTURER’S TECHNICAL SERVICE

8. WARRANTY POLICY

9. RECOMMENDED SERVICE TIMES

10. APPENDICES

10.1 General assembly drawing

10.2 Motor subassembly drawing

10.3 Mechanism assembly drawing

10.4 LH-RH Housing assembly drawing

10.5 Conrod-Leaf Spring assembly

1. INTRODUCTION 
This document provides information and recommendations for the maintenance and servicing of the AS170 by qualified service centers.

To ensure correct repairs, only use ARBORTECH factory supplied spares or other components as specified here in.

2. SAFETY

WARNING! READ AND UNDERSTAND ALL INSTRUCTIONS. FAILURE TO FOLLOW ALL INSTRUCTIONS LISTED BELOW MAY RESULT IN ELECTRIC SHOCK, FIRE AND/OR SERIOUS PERSONAL INJURY.

Refer to the Operations Instruction manual for guidelines on safety and the general use of this tool. The following additional safety guidelines are recommended to be observed in performing detailed servicing and troubleshooting of the tool.

CAUTION: ALWAYS WEAR THE APPROPIATE SAFETY GEAR WHEN SERVICING ARBORTECH TOOLS.

- Take care when servicing electrical items to ensure shock hazards are eliminated.

- Use care when handling components of the tool during or after use as some items become hot.

- Do not operate tool without all electrical covers in place?

WARNING:
BEFORE ATTEMPTING ANY SERVICE WORK ENSURE THAT THE
TOOL IS DISCONNECTED FROM ANY POWER SOURCE

3. SPECIFICATIONS

3.1 ELECTRICAL

| ITEM | AU/EU/UK – 230V | USA/UK – 110V |
| --- | --- | --- |
| Noise level | 92dB | 92dB |
| Operating voltage, nominal | 230-240VAC 50 Hz | 110-120VAC 50-60 Hz |
| Rated power consumption | 1250W | 1250W |
| Current draw of tool, free running without blades | 4.1 Amp | 7.9 Amp |
| Rotor armature resistance, nominal | 1.75Ω±6% | 0.49Ω±6% |
| Stator resistance, nominal (each) | 1.16Ω±6% | 0.31Ω±6% |

3.2 MECHANICAL

| ITEM | DESCRIPTION |
| --- | --- |
| Conrod orbit rate, free running without blades | 5100 Rpm |
| Belt tension | 1.5mm/ 1/16” maximum mid-span deflection with 25 N (2.5kg/8.5lb) deflection force |
| Blade bolt size | M6x10mm Cap Screw with chamfered head. |
| Blade contact and pre-load force | Blades must only contact each other at the periphery and with a light force, i.e. less than approximately 3 N (0.3kg) |
| Gearbox grease | Type Castrol APX, 15-20cc |
| Torque settings: |  |
| Blade bolt torque | 16Nm (11.5ft-lb) |
| - Leaf spring Conrod bolt torque - Leaf spring housing bolt torque | 20Nm (14.75ft-lb) 20Nm (14.75ft-lb) |
| Plastic tapping screw torque | 2Nm (1.5ft-lb) |
| - Motor pinion gear nut torque - Housing bolt torque | 6Nm (4.4ft-lb) 6Nm (4.4ft-lb) |

4. DUST EXTRACTION ACCESSORY

The dust extraction accessory: 30mm / 1 3/16” inside-diameter.

Dust extraction fitting: Suits vacuum hoses inner diameter of: 25mm (1”), 32mm (1-1/4”), 38mm (1-1/2”) or outer hose diameter 32mm (1-1/4”)

5. TOOLS REQUIRED

| ITEM | DESCRIPTION |
| --- | --- |
| Multimeter | Preferably with capacitance measurement |
| Torque wrench: | 2 Nm, w/cross-point screwdriver attachment 6 - 22 Nm, +/- 10% accuracy, w/hex key attachment 6 Nm, w/hex socket attachment |
| Bearing press | Small hand operated, bench mounted – suggest 1 ton arbor press |
| Bearing puller | Internal, 9 - 12 mm / 3/8-1/2” ID |
| Belt tensioner (typical spring scale) capable of: | 25 N force |

6.1 BELT COVER REMOVAL AND INSTALLATION

To remove the belt cover, loosen the screws holding the plastic cover onto the right side of the tool.

To reinstall the belt cover, screw the cover back onto the aluminium housing and verify that the cover does not contact the belt or pulleys.

6.2 REAR HANDLE COVER REMOVAL AND INSTALLATION

Remove the Brush doors, and then undo the seven screws that hold the handle together. Take top half of handle off to expose switch and electrical wires.

NOTE
DO NOT REMOVE THE RATING LABEL FROM THE SIDE OF 
THE MOTOR HOUSING. REMOVAL WILL VOID WARRANTY.

Install top half of handle back over switch and electrics and tighten the seven screws firmly. Replace Brush Doors.

6.3 ALUMINIUM HOUSING SEPARATION AND RE-ASSEMBLY

Remove blades

Remove the wire guard from the front of the tool.

Remove the top handle by removing the pins which are held in place with a small internal circlip.

Separate the left housing from the right housing by removing the bolts which hold them together. To separate the housings, tap using a mallet at the various openings in the split line, i.e. handle and conrod openings.

NOTE
USE CARE WHEN SEPARATING THE HOUSINGS AS EACH SIDE
SUPPORTS ONE END OF THE CROWNSHAFT AND CAMSHAFT.

The bearings for the left side of the crownshaft and camshaft are pressed into the left housing and should remain with the housing when it is removed.

HINT
TO PREVENT THE GEARBOX GREASE FROM BECOMING CONTAMINATED DURING
MAINTENANCE, COVER THE GEARBOX OPENING WITH ADHESIVE TAPE OR SIMILAR.

After verifying the mating surfaces of each housing are clean and not nicked or damaged, re-fit the left aluminium housing to the right, ensuring that the alignment pins and mating surfaces come together correctly. (Carefully dress any nicked or damaged areas on the mating surfaces). Tighten the bolts as per the specifications.

6.4 CLEANING OF COOLING VENTS

Cleaning the cooling vents simply by using an air blast from the outside of the tool is somewhat effective but could dislodge debris into the motor/stator gap and potentially cause binding. It is therefore recommended that the rear cover/handle cover of the motor be removed as described in this document.

Use an airblast to clean the rear cover/handle cover (from the inside out) and the switch area. Avoid dislodging debris into the stator/rotor area.

View the commutator and rotor area for obstructions to airflow. Clear any obstructions with air blasts taking care not to force debris into the motor and checking to verify they have been cleared. If they cannot be cleared it will be necessary to further disassemble the motor or housing depending on where the obstruction lies. Refer to the appropriate sections of this manual and/or our Service DVD for instructions on accessing these areas.

IMPORTANT
IT IS CRITICAL TO ENSURE THAT ANY BUILDUP OF DEBRIS ON THE ROTOR AND STATOR IS
REMOVED AS ANY CONTACT BETWEEN THEM WILL CAUSE OVERHEATING FAILURE

6.5 BELT INSPECTION, TENSIONING AND REPLACEMENT

Refer to the Operators Instruction Manual and/or Service DVD.

6.6 PULLEY INSPECTION AND REPLACEMENT

Refer to the Operators Instruction Manual and/or Service DVD.

6.7 MOTOR COMMUTATOR BRUSH INSPECTION AND REPLACEMENT

Remove the brush covers, pull the small spiral spring back clear from the top of the brush, you will then be able to remove the brush from the holder. The brush arm should be inspected for burning and if any signs of such burning is found, they should be replaced.

6.8 MOTOR INSPECTION, REPAIR OR REPLACEMENT

A suspected motor problem can initially be partially diagnosed by simply checking the current draw and the camshaft speed with the tool free-running (without blades attached). Refer to the specifications for acceptable limits. Assuming the bearings and other components are not causing excessive load, then if the motor is drawing too much current or the camshaft is turning too slowly, the motor is suspect.

If the motor is operating out of specifications perform the following additional simple checks (refer to the relevant section) to try to more specifically determine the cause, by verifying:

- Brushes not excessively worn

- Commutator not burned, worn, or damaged

- Switch operating normally

If none of the above items are faulty, an internal inspection of the motor is required. The left side aluminium

housing and belt cover must be removed to access the screws holding the motor to the housing. (Refer to the relevant section 6.3 in this document).

NOTE
THE REAR HANDLE COVER MUST BE REMOVED, AND THE BRUSHES REMOVED AS FOR BRUSH
INSPECTION AND REPLACEMENT

All 4 PT screws holding the right housing to the plastic motor casing will now be accessible and need to be removed.

Carefully pull the motor away from the aluminium housing. This will meet with some resistance as the front motor bearing is a firm fit in the aluminium casting. It may require use of a mallet to tap it off.

To remove the rotor from the motor casing, open the rear handle covers up to expose the switch and brush gear. Unscrew the rear fan from the rotor shaft; with it removed you will be able to pull the rotor out.

Inspect the rotor and stator for signs of overheating, short-circuiting or broken wires. Note that there should be no signs of deformation of either winding as this can cause contact between moving parts and overheating. Clean any debris or other buildup which could cause contact or impede cooling air flow. Either item which appears abnormal is cause for replacement with a new motor.

NOTE
IT IS RECOMMENDED THAT THE COMPLETE MOTOR BE REPLACED IF 
EITHER ROTOR OR STATOR IS FOUND TO BE UNSERVICEABLE.

Re-install the rotor and rear fan. Re fit rear handle covers.

Re-fit the motor housing to the aluminium housing using the PT screws ensuring that the screws are done up tightly.

IMPORTANT
IT IS CRITICAL THAT THE CORRECT PT SCREWS ARE USED AND TIGHTENED
AS SPECIFIED, OTHERWISE STRIPPING OF THREADS IN THE
MOTOR HOUSING, OR LOOSENING MAY OCCUR.

Re-install the commutator brushes.

Inspect the gearbox grease for contamination, insufficient volume or other signs of problems and replace or top-up as required. The type and total volume of grease must be per the specifications. Replace the O-ring gearbox seal if needed.

Inspect and replace the top handle shock isolation bushes as required.

Re-fit the left aluminium housing to the right, as described in this document, and re-install the wire guard.

6.9 CONROD ASSEMBLY REPLACEMENT

Should either of the bearings on the camshaft or conrod require inspection or replacement, the camshaft pulley needs to be removed and the aluminium housing separated. Refer to the relevant sections in this document.

With the tool lying on the right side, loosen the bolts securing the rear ends of the leaf springs to the aluminium housing.

Slide the camshaft and conrod assembly out of the bearing on the right housing.

Determine whether the bearings in the aluminium housing are faulty by rotating each manually to detect any sign of roughness or lack of lubricant. If faulty, use a bearing puller to remove. (They are lightly press fitted).

Take care to ensure the bearing cavities in the housings are clean and install new bearings using a bearing press, paying special attention to ensure that they are pressed squarely. Verify that they are fully seated.

Using a solvent or small plastic scraper, clean any adhesive or other debris from both the leaf spring ends and the seating area on the aluminium housing.

Apply LOCTITE 243 to bolts and mating faces. Install the leaf spring shims, bolts and lockwashers per the assembly drawing. Do not torque spring mounting bolts at this stage.

Follow the relevant instructions for re-assembly of the aluminium housing.
Torque spring mounting bolts to specifications after reassembling the aluminium housing.

6.10 CROWNSHAFT BEARING REPLACEMENT

Should either of the crownshaft bearings require inspection or replacement the aluminium housing must be separated and the gearshaft pulley removed. Refer to the relevant sections in this document.

With the tool laying on its right side, the crownshaft with crown gear can be lifted out of the gearbox. It may need to be tapped out with a soft mallet.

Determine whether the bearings in the aluminium housing are faulty by rotating each manually to detect any sign of roughness or lack of lubricant. If faulty, replace them by removing with a bearing puller and installing new bearings squarely with bearing press.

While the gears are accessible, check the condition of the crown and pinion gears and grease, and replace as required. Refer to the relevant sections of this document.

Re-assemble the aluminium housings (also replacing the gearbox O-ring if necessary), per the relevant sections in this document.

Assemble the pulley and manually verify that the gearshaft rotates smoothly. In some cases, when the tool is reassembled, the gear mesh may be tight causing it to bind with the motor pinion gear. To remedy this, lightly tap on the centre of the gearshaft pulley with a plastic or wooden mallet. Re-check that the gearshaft turns freely by hand.

Finally, assemble the belt and cover, and verify that the tool runs smoothly. Verify it is running at the specified speed, without the blades mounted.

6.11 GEAR INSPECTION AND REPLACEMENT

Remove the crownshaft pulley and separate the aluminium housing per the relevant sections of this document.
Remove the crownshaft with crown gear and inspect the motor pinion gear for signs of wear or damage.
The gear teeth should have little or no visually detectable wear, should have a polished finish along the
meshing line and no chips or other faults. If either gear appears worn or damaged both gears must be replaced.
Replace crown gear and shaft with a new assembly from the manufacturer. To replace the pinion gear the motor must be removed. Refer to the relevant section of this document. Once the pinion gear is accessible, the nut can be loosened and the gear replaced, tightening the nut to the torque specified.

NOTE
THE ROTOR SHOULD ONLY BE HELD WITH "SOFT JAWS" WITH MINIMAL 
CLAMPING FORCE, TO AVOID DAMAGING THE ARMATURE LAMINATIONS.

Replace the gearbox grease with new grease in the volume specified.
Re-assemble the motor to the aluminium housing and torque the PT screws as specified.

NOTE
ONLY PT SCREWS MEETING THE MANUFACTURER’S SPECIFICATIONS SHOULD BE USED.

Slide the crownshaft into the gearbox and reassemble the aluminium housing as described in the relevant section of this document.

Install the crownshaft pulley and check for free and smooth rotation of the gearshaft. If not, follow the instructions described in the gearshaft bearing replacement procedure.

Re-assemble the belt and cover and run the tool to verify correct operation and free running speed.

6.12 POWER CORD INSPECTION AND REPLACEMENT

Inspection of the power cord consists of a visual check for damage, including less obvious defects such as pinching or damage which could have occurred internally with little external evidence. Additionally, the cord and plug should be checked for continuity and internal isolation resistance.

Remove the rear handle cover as described in this document.

Unscrew the power cord wires from the motor switch.

Check and replace power cord as required.

6.13 BLADE INSPECTION AND MAINTENANCE

Note that blades are a consumable item and as such are not covered under warranty.

If blades are removed, do not operate the tool with the blade mount bolts in the conrods.

The blades should be inspected for fatigue cracks in the area near the mounting plate of the blade. With use, the blades will become dull and the cutting performance will decrease. This can be largely restored by re-sharpening the teeth, providing that the side clearance has not been significantly worn away. Refer to the sharpening instructions in the Operator’s Instruction Manual, noting that the tungsten-carbide tipped blades require a diamond tool for sharpening.

If "blueing" of the blade periphery occurs, the blades are running too hot and this can cause the Tungsten Carbide teeth to be lost. Overheating blades can be due to worn teeth, too much force being used, insufficient "sweeping" motion by the operator, or the material being too hard.

7. MANUFACTURER’S TECHNICAL SERVICE 
For technical inquiries and assistance contact:

Arbortech Industries 67 Westchester Rd Malaga, Perth Western Australia 6090

Arbortech Deutschland Hallenstrabe 13 Finnentrop 57413

Arbortech USA 80 Erdman Way, Suite 200 Leominster, MA, 01453

| Phone: | +61(8) 9249-1944 |  | US Ph: |  | (978) 466-7869 |
| --- | --- | --- | --- | --- | --- |
| Fax: | +61(8) 9249-2936 |  | US Fax: |  | (866) 515 8104 |
| Email: | customerservice@arbortech.com.au |  | US Email: |  | csr@arbortechusa.com |
| Website: | www.arbortech.com.au |  | Website: |  | www.arbortechusa.com |
| EU Ph: |  | 02724 – 288301 |  |
| EU Fax: |  | 02724 – 205515 |  |
| EU Email: |  | info@arbortech-europa.de |  |
| Website: |  | www.arbortech-europa.de |  |

8. WARRANTY POLICY

Every Arbortech AS170 Brick + Mortar Saw is thoroughly inspected and tested before leaving the factory.

12 months warranty on mechanism

Arbortech provides a 12 months warranty on the mechanism from the date of original purchase. This covers mechanical components* to be free of defects from workmanship and materials.

*Excluding consumables: Belt, blades, blade bolts, brushes, brush holders and dust boot.

12 months warranty on motor

Arbortech provides a 12 months warranty on the motor from the date of original purchase.

Please note that this warranty does not apply where:

- Unauthorized repairs have been carried out

- Repairs are required due to normal wear and tear

- Repairs are required to accessory products

- There is evidence of unauthorized alterations

- The tool has been abused, altered or improperly maintained

- If the Product has not been used in accordance with the manufacturer’s instructions or specifications provided with the Product

Arbortech makes no warranties, representations, or promises as to the quality or performance of the AS170 Brick + Mortar Saw other than those specifically stated in this warranty or Operators Instruction Manual.

Any implied warranty granted under state law, including warranties of merchantability or fitness for a particular purpose, are limited to the specified times stated above. Arbortech is not responsible for direct, indirect, incidental or consequential damages.

This warranty does give you specific legal rights, and you may also have other rights that may vary from state to state.

Should any trouble develop during this one-year period, return the COMPLETE tool, to one of Arbortech’s Factory or Authorized Service Centers.

Freight

In some cases, Arbortech will accept responsibility for the payment of freight to and from your chosen delivery address. If the repairs are not covered by warranty then freight charges will be recharged to the customer.

9. RECOMMENDED SERVICE TIMES

| RECOMMENDED SERVICE TIMES FOR AS170 BRICK + MORTAR SAW |  |  |
| --- | --- | --- |
| ITEM | TIME |
| Handle bushes | 10 mins |
| Switch replacement | 10 mins |
| Cord replacement | 10 mins |
| Brush replacement | 10 mins |
| Belt replacement | 10 mins |
| Shaft & crown gear | 20 mins |
| Motor change | 30 mins |
| Camshaft & springs assembly | 30 mins |
| Housing bearing replacement | 30 mins |

| AS170 SPARE PARTS PRICE LIST AND REF. DIAGRAM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| KEY # | PART # | DESCRIPTION | Model | # in Pack. |
| 1 | ALL.FG.SCREW015 | SCREW, BELT COVER M4 X 12 | 160/170 | 4 |
| 2 | ALL.FG.BOLT17012 | BOLT, BLADE BEVELLED M6 X 10 | 160/170 | 4 |
| 3 | ALL.FG.BRUSH1706 | BRUSH COVER, ELECTRIC | 170 | 1 |
| 4 | ALL.FG.WASH024 | WASHER , STEP M6 | 160/170 | 1 |
| 5 | ALL.FG.NUT006 | NUT M6 NYLOC | 160/170 | 1 |
| 6 | ALL.FG.BRUSH1703 | BRUSH CARBON 240V SET PKT 2 | 170 | 2 |
| 8 | ALL.FG.CLIP001 | CLIP, INTERNAL M8 | 160/170 | 2 |
| 9 | ALL.FG.PIN002 | PIN, HANDLE | 160/170 | 2 |
| 10 | ALL.FG.TENSION001 | TENSION PULLEY& BEARING | 160/170 | 1 |
| 11 | ALL.FG.PULLEY005 | PULLEY, POWER | 160/170 | 1 |
| 12 | ALL.FG.PULLEY004 | PULLEY, DRIVEN | 160/170 | 1 |
| 13 | ALL.FG.BELT002 | BELT | 160/170 | 1 |
| 14 | ALL.FG.BUSH001 | BUSH, RUBBER SET | 160/170 | 4 |
| 15 | ALL.FG.GUARD002 | GUARD, WIRE | 160/170 | 1 |
| 16 | ALL.FG.HANDLE001 | HANDLE, TOP | 160/170 | 1 |
| 17 | ALL.FG.CONROD003 | CONRODS & CAM ASSEMBLY | 160/170 | 1 |
| 18 | ALL.FG.SPRINGASS001 | SPRING ASSEMBLY | 160/170 | 1 |
| 19 | ALL.FG.CONROD001 | CONROD, ASSEMBLY | 160/170 | 1 |
| 20 | ALL.FG.COVER001 | COVER, BELT | 160/170 | 1 |
| 21 | ALL.FG.HOUSE170LH | HOUSING, LEFT HAND AS170 | 170 | 1 |
| 22 | ALL.FG.HOUSE170RH | HOUSING RIGHT HAND AS170 | 170 | 1 |
| 23 | ALL.FG.MOTOR1703.60 | MOTOR, AS170 QS 1250W 240V | 170 | 1 |
| 25 | ALL.FG.CORD170.xx | CORD, AS170 - 230V | 170 | 1 |
| 26 | ALL.FG.BOOT001 | BOOT, DUST EXTRACTOR | 160/170 | 1 |
| 27 | ALL.FG.BRACKET003 | BRACKET, DUST EXTRACTION FITTING | 160/170 | 1 |
| 28 | ALL.FG.BRUSH1705 | BRUSH, HOLDER SET PKT 2 | 170 | 2 |
| 29 | ALL.FG.SWITCH1702 | SWITCH 110/240V | 170 | 1 |
| 32 | ALL.FG.SEAL17001 | SEAL AS170, O-RING M52 X 2 | 170 | 1 |
| 33 | ALL.FG.BEARING002 | BEARING CAM SHAFT 6001 | 160/170 | 1 |
| 34 | ALL.FG.BEARING004 | BEARING, RH CROWN 629 | 160/170 | 1 |
| 35 | ALL.FG.CROWN170 | CROWN SHAFT ASSEMBLY AS170 | 170 | 1 |
| 36 | ALL.FG.BEARING002 | BEARING CAM SHAFT 6001 | 160/170 | 1 |
| 37 | ALL.FG.BEARING006 | BEARING, LH CROWN 6201 | 160/170 | 1 |
| 38 | ALL.FG.SCREW011 | SCREW, HOUSING M5 X 25 | 160/170 | 5 |
| 39 | ALL.FG.HANDLE17002 | REAR HANDLE - BOTH HALVES | 170 | 1 LH/1 RH |
| 40 | ALL.FG.SCREW016 | SCREW, OUTER FIELD CASING PT4x22 | 160/170 | 4 |
| 41 | ALL.FG.PIN001 | PIN, DOWELL, M3 X 16 | 160/170 | 1 |
| 42 | ALL.FG.BOLT004 | BOLT M6 X 20 | 160/170 | 1 |
| 44 | ALL.FG.ADAPT001 | VACUUM ADAPTOR | 160/170 | 1 |
| 45 | ALL.FG.BAG170 | BAG, AS170 TOOL | 160/170 | 1 |
| 46 | ALL.FG.KEY004 | KEY, TORQUE 16NM | 160/170 | 1 |
| 47 | ALL.FG.FIELDCASE170 | FIELD CASING | 170 | 1 |
| 51 | ALL.FG.BEARING005 | BEARING, CONROD 6004 | 160/170 | 1 |
| 52 | ALL.FG.BEARING007 | BEARING, FRONT MOTOR 608-05 | 160/170 | 1 |
| 53 | ALL.FG.BEARING008 | BEARING, REAR MOTOR 607-05 | 160/170 | 1 |
| 54 | ALL.FG.CLIP002 | CIRCLIP INTERNAL 42 X 1.75 | 160/170 | 1 |